# ESP32 RoboSoccer Bot — Complete Beginner-to-Build Guide

### Prayas 4.0 — Robotics, AI & IoT Lab | IIT Mandi

## How This Document Is Organized

Part A — ESP32 Fundamentals (First-Time Setup) You will learn what the ESP32 is, how to set up Arduino IDE for it, upload your first program, talk to it over Serial, connect it to WiFi (both as a client and as its own access point), briefly understand Bluetooth on the ESP32, and host a simple webpage from the board that toggles an LED and shows live ultrasonic sensor readings.

Part B — RoboSoccer Bot Build Once you're comfortable with the ESP32, we build a full 2-motor + 2-servo RoboSoccer bot controlled entirely from a webpage hosted on the ESP32 itself, accessed from your mobile phone browser — no app installation needed.

No prior knowledge of ESP32 is assumed. Every step, every button, every wire is explained.

# PART A — ESP32 FUNDAMENTALS

## A.1 What Is an ESP32?

The ESP32 is a small microcontroller board — think of it as a tiny computer that can run one program at a time, read sensors, and control motors/LEDs, but unlike Arduino Uno, it has built-in WiFi and Bluetooth. This is exactly why we can host a webpage directly on it and control our bot wirelessly without any external WiFi module.

The board you're holding is likely an ESP32 DevKit V1 (sometimes called ESP32 DevKitC or NodeMCU-32S). It looks like a long rectangular board with:

- A USB Micro-B (or USB-C on newer variants) port on one end

- Two rows of pins running down both long edges

- A metal-shielded chip (the ESP32 chip itself) roughly in the middle

- Two small push buttons near the USB port

## A.2 Know Your Board — The Two Buttons

This trips up almost every beginner, so let's be very clear:

| Button | Label (varies by board) | What it does |
| --- | --- | --- |
| EN (or RST/RESET) | Usually unlabeled or "EN", closer to the antenna | Resets the chip. Pressing it restarts whatever program is currently on the board — same as power-cycling it. |
| BOOT (or IO0) | Usually labeled "BOOT", closer to the USB port | Forces the chip into "download/flash mode" when held during reset. Needed on SOME boards to upload code. |

Why this matters: Most ESP32 DevKit boards have auto-reset circuitry, so Arduino IDE can upload code without you touching any buttons at all. But sometimes — especially on cheaper clone boards, or the first time you use a fresh board — the upload will fail with a message like Connecting........_____ and time out. When this happens:

- Click Upload in Arduino IDE.

- As soon as you see the "Connecting..." message in the console, press and hold BOOT.

- While still holding BOOT, tap EN/RESET once and release it.

- Keep holding BOOT for another second or two, then release it.

- The upload should now proceed.

Just remember: BOOT = "let me talk to you in programming mode", EN/RESET = "restart". You will use this trick occasionally throughout this course, so don't worry if it doesn't make full sense yet — you'll get muscle memory for it fast.

## A.3 Installing Arduino IDE

- Go to https://www.arduino.cc/en/software on your laptop.

- Download the Arduino IDE for your OS (Windows/Mac/Linux). Use the latest 2.x version.

- Install it like any normal application (Next → Next → Install, or drag to Applications on Mac).

- Open Arduino IDE once to confirm it launches correctly. You'll see a blank sketch with void setup() {} and void loop() {} — this is normal, don't touch it yet.

## A.4 Adding ESP32 Board Support to Arduino IDE

Arduino IDE does not know about the ESP32 out of the box — we need to add it via the Boards Manager URL.

Step-by-step:

- Open Arduino IDE.

- Go to File → Preferences (on Mac: Arduino IDE → Settings).

- Find the field labeled "Additional Boards Manager URLs".

- Paste this exact URL into that box:

https://raw.githubusercontent.com/espressif/arduino-esp32/gh-pages/package_esp32_index.json

If there is already a URL in that box from a previous install, don't delete it — click the small icon next to the field to open a list editor, and add this as a new line/entry.

- Click OK.

- Now go to Tools → Board → Boards Manager... (or the boards icon on the left sidebar that looks like a circuit board).

- In the search box, type esp32.

- You'll see an entry called "esp32 by Espressif Systems". Click Install (this may take a few minutes and download ~400MB — needs internet).

- Once installed, close the Boards Manager.

Selecting your board:

- Plug your ESP32 into your laptop via USB cable.

- Go to Tools → Board → esp32 → "ESP32 Dev Module". (If you're not sure of your exact variant, "ESP32 Dev Module" works for almost all standard DevKit boards.)

- Go to Tools → Port, and select the port that appeared after you plugged in the board.

  - Windows: something like COM3, COM5

  - Mac: something like /dev/cu.usbserial-0001 or /dev/cu.SLAB_USBtoUART

  - Linux: something like /dev/ttyUSB0

  - If no new port appears, you likely need USB drivers — see A.5 below.

## A.5 Driver Installation (If Your Board Isn't Detected)

Most ESP32 DevKit boards use either a CP2102 or CH340 USB-to-Serial chip. If your board doesn't show up in Tools → Port after plugging in:

- Look at the small chip near the USB port on your ESP32 board — it will have "CP2102" or "CH340" printed on it.

- Download the matching driver:

  - CP2102: search "Silicon Labs CP210x driver" — official Silicon Labs download page.

  - CH340: search "CH340 driver" — install the version for your OS.

- Install the driver, restart your laptop, then reconnect the ESP32 and check Tools → Port again.

## A.6 Activity 1 — Blink Code (Your First Upload)

The ESP32 DevKit has a built-in LED on the board itself (usually on GPIO pin 2, sometimes labeled "LED" next to it — a few clone boards use GPIO 4 or don't have one, in which case we'll wire an external LED).

Code:

// Activity 1: Blink the onboard LED

#define LED_PIN 2   // change to 4 if your board's onboard LED doesn't respond

void setup() {

pinMode(LED_PIN, OUTPUT);

}

void loop() {

digitalWrite(LED_PIN, HIGH);  // LED ON

delay(1000);                  // wait 1 second

digitalWrite(LED_PIN, LOW);   // LED OFF

delay(1000);                  // wait 1 second

}

How to upload:

- Copy this code into a new sketch (File → New Sketch), replacing whatever is there.

- Click the checkmark (✓) icon top-left to Verify/Compile — this checks for errors without uploading.

- Click the right-arrow (→) icon next to it to Upload.

- Watch the bottom console — you'll see dots and eventually "Hard resetting via RTS pin..." or "Done uploading."

- If it hangs at "Connecting...", use the BOOT+EN trick from Section A.2.

- Once uploaded, your onboard LED should start blinking once every second.

If nothing happens: double-check the board and port are correctly selected in Tools menu, and try the BOOT button trick.

## A.7 Activity 1b — Serial Communication

Now let's make the ESP32 "talk" back to your laptop through the USB cable using the Serial Monitor. This is essential for debugging everything from here on.

// Activity 1b: Blink + Serial print

#define LED_PIN 2

void setup() {

pinMode(LED_PIN, OUTPUT);

Serial.begin(115200);   // start serial communication at 115200 baud

Serial.println("ESP32 Serial started!");

}

void loop() {

digitalWrite(LED_PIN, HIGH);

Serial.println("LED ON");

delay(1000);

digitalWrite(LED_PIN, LOW);

Serial.println("LED OFF");

delay(1000);

}

Steps:

- Upload this code (same steps as before).

- Open Tools → Serial Monitor, or click the magnifying-glass icon top-right.

- Important: In the bottom-right corner of the Serial Monitor window, set the baud rate dropdown to 115200 — this must match Serial.begin(115200) in the code, or you'll see garbled text/nothing.

- You should now see "LED ON" and "LED OFF" printing once per second, in sync with the physical LED blinking.

Serial.begin(115200) starts communication at 115200 bits per second — you can think of baud rate as "how fast the ESP32 and your laptop agree to talk." Both sides must match.

## A.8 Connecting ESP32 to WiFi (Station Mode — Using Your Mobile Hotspot)

"Station mode" means the ESP32 connects to an existing WiFi network — in our case, your mobile phone's hotspot — just like your laptop does.

Step 1: Turn on your phone's mobile hotspot.

- Android: Settings → Network & Internet → Hotspot & Tethering → Wi-Fi Hotspot → turn on. Note the Hotspot name (SSID) and password shown there.

- iPhone: Settings → Personal Hotspot → turn on. Note the WiFi Password shown.

Step 2: Upload this code, editing the two lines marked CHANGE THIS:

#include <WiFi.h>

const char* ssid     = "YOUR_HOTSPOT_NAME";     // CHANGE THIS to your phone hotspot name

const char* password = "YOUR_HOTSPOT_PASSWORD"; // CHANGE THIS to your phone hotspot password

void setup() {

Serial.begin(115200);

delay(1000);

Serial.print("Connecting to ");

Serial.println(ssid);

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("");

Serial.println("WiFi connected!");

Serial.print("ESP32 IP address: ");

Serial.println(WiFi.localIP());

}

void loop() {

}

Where exactly to edit:

- Replace YOUR_HOTSPOT_NAME with your phone's hotspot name, e.g. const char* ssid = "Rahuls_Phone";

- Replace YOUR_HOTSPOT_PASSWORD with your hotspot password, e.g. const char* password = "12345678";

- Keep the quotation marks — only change the text inside them.

After uploading: open Serial Monitor (baud 115200). You should see dots, then "WiFi connected!" followed by an IP address like 192.168.43.150. Write this down — it's how your phone's browser finds the ESP32, and it can change each session so always re-check via Serial Monitor.

Common issue: ESP32 only supports 2.4GHz WiFi, not 5GHz. If your hotspot is 5GHz-only, switch it to a compatible/2.4GHz mode.

## A.9 ESP32's Own WiFi (Access Point / AP Mode)

Instead of joining your phone's hotspot, the ESP32 can create its own WiFi network that your phone connects to directly. This is what we use for the final RoboSoccer bot, since a soccer field may not have reliable hotspot range — the bot creates its own WiFi bubble.

#include <WiFi.h>

const char* ap_ssid     = "RoboSoccer_Bot";   // CHANGE THIS if you want a different bot name

const char* ap_password = "12345678";          // CHANGE THIS - must be at least 8 characters

void setup() {

Serial.begin(115200);

delay(1000);

WiFi.softAP(ap_ssid, ap_password);

Serial.println("Access Point started!");

Serial.print("Connect your phone to WiFi network: ");

Serial.println(ap_ssid);

Serial.print("ESP32 AP IP address: ");

Serial.println(WiFi.softAPIP());

}

void loop() {

}

Steps:

- Upload this code.

- Open Serial Monitor — note the IP printed (almost always 192.168.4.1).

- On your phone, go to WiFi settings and connect to "RoboSoccer_Bot" using password 12345678.

- Your phone is now directly connected to the ESP32's own WiFi — no internet, no router needed.

Remember the difference:

- Station mode (A.8): ESP32 joins an existing network. Good when the ESP32 also needs internet access.

- AP mode (A.9): ESP32 creates its own network. Good for standalone/field robots.

## A.10 A Quick Word on Bluetooth

The ESP32 also supports Bluetooth (Classic and BLE). We are not using Bluetooth in this project — WiFi + a hosted webpage gives us buttons, sliders, and live sensor readouts directly in a browser, with no app install needed, whereas Bluetooth typically requires a dedicated terminal app. For awareness only (do not upload this):

// FYI only, not used in this project

#include "BluetoothSerial.h"

BluetoothSerial SerialBT;

void setup() {

Serial.begin(115200);

SerialBT.begin("ESP32_BT_Test");

}

void loop() {

if (SerialBT.available()) {

Serial.write(SerialBT.read());

}

}

## A.11 Activity 2 — Hosting a Webpage on ESP32 (LED Toggle)

This is the core trick behind the RoboSoccer control system: the ESP32 runs a tiny web server, and your phone's browser talks to it just like it talks to any website — except this "website" lives inside the ESP32 itself.

We use AP mode (A.9) so your phone connects directly to the bot.

#include <WiFi.h>

#include <WebServer.h>

const char* ap_ssid     = "ESP32_LED_Test";  // CHANGE THIS if you like

const char* ap_password = "12345678";         // CHANGE THIS - min 8 characters

#define LED_PIN 2

WebServer server(80);   // web server on port 80 (standard web port)

String htmlPage() {

String html = "<!DOCTYPE html><html><head><title>ESP32 LED Control</title>";

html += "<meta name='viewport' content='width=device-width, initial-scale=1'>";

html += "<style>body{font-family:Arial;text-align:center;margin-top:50px;}";

html += "button{padding:20px 40px;font-size:24px;margin:10px;border-radius:10px;border:none;}";

html += ".on{background-color:#4CAF50;color:white;}";

html += ".off{background-color:#f44336;color:white;}</style></head><body>";

html += "<h1>ESP32 LED Control</h1>";

html += "<p><a href=\"/led_on\"><button class='on'>LED ON</button></a></p>";

html += "<p><a href=\"/led_off\"><button class='off'>LED OFF</button></a></p>";

html += "</body></html>";

return html;

}

void handleRoot() {

server.send(200, "text/html", htmlPage());

}

void handleLedOn() {

digitalWrite(LED_PIN, HIGH);

server.send(200, "text/html", htmlPage());

}

void handleLedOff() {

digitalWrite(LED_PIN, LOW);

server.send(200, "text/html", htmlPage());

}

void setup() {

Serial.begin(115200);

pinMode(LED_PIN, OUTPUT);

WiFi.softAP(ap_ssid, ap_password);

Serial.print("Connect phone to WiFi: ");

Serial.println(ap_ssid);

Serial.print("Then open this in browser: http://");

Serial.println(WiFi.softAPIP());

server.on("/", handleRoot);

server.on("/led_on", handleLedOn);

server.on("/led_off", handleLedOff);

server.begin();

Serial.println("Web server started!");

}

void loop() {

server.handleClient();   // must run repeatedly to check for browser requests

}

Steps:

- Upload the code.

- On your phone, connect to WiFi network "ESP32_LED_Test" password 12345678.

- Open a browser app on your phone.

- Type 192.168.4.1 in the address bar (confirm exact IP via Serial Monitor).

- You'll see two buttons: LED ON / LED OFF. Tap them — the onboard LED responds in real time.

How it works: each button tap sends a small request to the ESP32 (e.g. "go to /led_on"). server.on("/led_on", handleLedOn) means "when someone asks for /led_on, run handleLedOn." That function flips the LED and sends the page back. This request-response pattern is the backbone of everything from here on, including motor control later.

## A.12 Activity 3 — Hosting Live Ultrasonic Sensor Readings

Now combine a sensor with the webpage — the HC-SR04 ultrasonic distance sensor, also used on the RoboSoccer bot.

HC-SR04 has 4 pins: VCC, Trig, Echo, GND.

Wiring:

| HC-SR04 pin | ESP32 pin |
| --- | --- |
| VCC | 5V (VIN pin) |
| GND | GND |
| Trig | GPIO 5 |
| Echo | GPIO 18 |

Voltage warning: HC-SR04's Echo pin outputs 5V, but ESP32 GPIOs are only safe up to 3.3V. For quick bench testing this is often tolerated briefly, but the correct practice is a voltage divider (e.g. 1kΩ + 2kΩ resistors) between Echo and GPIO18 to bring it down to ~3.3V. We do this properly in the RoboSoccer build in Part B.

#include <WiFi.h>

#include <WebServer.h>

const char* ap_ssid     = "ESP32_Ultrasonic_Test";

const char* ap_password = "12345678";

#define TRIG_PIN 5

#define ECHO_PIN 18

WebServer server(80);

long readDistanceCM() {

digitalWrite(TRIG_PIN, LOW);

delayMicroseconds(2);

digitalWrite(TRIG_PIN, HIGH);

delayMicroseconds(10);

digitalWrite(TRIG_PIN, LOW);

long duration = pulseIn(ECHO_PIN, HIGH, 30000); // 30ms timeout

long distance = duration * 0.034 / 2;  // speed of sound = 0.034 cm/microsecond

return distance;

}

void handleRoot() {

String html = "<!DOCTYPE html><html><head><title>Ultrasonic Reading</title>";

html += "<meta name='viewport' content='width=device-width, initial-scale=1'>";

html += "<meta http-equiv='refresh' content='1'>";

html += "<style>body{font-family:Arial;text-align:center;margin-top:50px;}";

html += "h1{font-size:60px;color:#2196F3;}</style></head><body>";

html += "<h2>Distance Reading</h2>";

html += "<h1>" + String(readDistanceCM()) + " cm</h1>";

html += "</body></html>";

server.send(200, "text/html", html);

}

void setup() {

Serial.begin(115200);

pinMode(TRIG_PIN, OUTPUT);

pinMode(ECHO_PIN, INPUT);

WiFi.softAP(ap_ssid, ap_password);

Serial.print("Then open this in browser: http://");

Serial.println(WiFi.softAPIP());

server.on("/", handleRoot);

server.begin();

}

void loop() {

server.handleClient();

}

Steps:

- Wire HC-SR04 as per the table above.

- Upload, connect phone to ESP32_Ultrasonic_Test, open 192.168.4.1 in browser.

- The page shows a live-ish distance reading, auto-refreshing every second.

- Wave your hand in front of the sensor and watch the number change.

This completes Part A. You now know: board buttons, Arduino IDE + ESP32 setup, uploading code, Serial communication, WiFi station and AP mode, Bluetooth awareness, and hosting a webpage that controls an output and displays live sensor data — exactly what the RoboSoccer bot needs.

# PART B — ROBOSOCCER BOT BUILD

## B.1 What We're Building

A 2-wheel-drive robot (differential drive) with a front castor wheel for balance, powered by two DC motors through an L298N motor driver, with two servo motors (for a kicking mechanism / flipper — commonly used in robosoccer to strike the ball), and ultrasonic sensor(s) for obstacle/ball proximity sensing. Everything is controlled from a webpage hosted on the ESP32 itself — your phone connects to the bot's own WiFi and gets a control panel with movement buttons, a kick button, and a live distance readout.

## B.2 Full Components List

| Component | Qty | Purpose |
| --- | --- | --- |
| ESP32 DevKit | 1 | Brain of the robot |
| L298N Motor Driver Module | 1 | Drives the 2 DC motors |
| BO/Geared DC Motors | 2 | Left and right wheel drive |
| Wheels (matching DC motors) | 2 |  |
| Castor wheel (ball caster) | 1 | Front support, allows free turning |
| SG90 Servo Motors | 2 | Kicker mechanism / flipper arms |
| HC-SR04 Ultrasonic Sensor | 1–2 | Ball/obstacle distance sensing |
| Robot chassis | 1 | Base plate to mount everything |
| Li-ion/LiPo battery pack or 4×AA holder (7.4V–9V) | 1 | Powers motors + ESP32 |
| Jumper wires (M-M, M-F, F-F) | ~30 | Wiring |
| Breadboard (optional) | 1 | For resistor voltage dividers |
| Resistors 1kΩ and 2kΩ | 2 each (per ultrasonic sensor) | Voltage divider for Echo pin(s) |
| Screws/standoffs, double-sided tape | — | Mechanical mounting |

## B.3 Understanding the L298N Motor Driver

DC motors need more current than a microcontroller pin can safely supply, and they need a way to spin in both directions. The L298N is a driver board that sits between the ESP32 (logic/control side) and the motors (power side), solving both problems.

L298N pin layout (typical board):

- Power side: 12V (or VMS) screw terminal — battery positive goes here. GND screw terminal — battery negative AND ESP32 GND both go here (this "common ground" step is critical, explained below). There's usually a small onboard voltage regulator with a jumper — if your battery is under 12V (like our 7.4V pack) you can normally leave the 5V-enable jumper in place to also power logic, but we will power ESP32 separately via USB/battery for reliability.

- Motor outputs: OUT1, OUT2 (Motor A / left motor), OUT3, OUT4 (Motor B / right motor).

- Control (logic) side: IN1, IN2 (direction control for Motor A), IN3, IN4 (direction control for Motor B), ENA (speed/enable for Motor A), ENB (speed/enable for Motor B).

How direction control works: for Motor A, IN1=HIGH, IN2=LOW spins it one way; IN1=LOW, IN2=HIGH spins it the other way; both LOW = motor stops (coasts); both HIGH = motor brakes. Same logic for IN3/IN4 with Motor B. ENA/ENB control speed via PWM (Pulse Width Modulation) — a faster on/off switching pattern that effectively controls average voltage, i.e. motor speed.

⚠️ CRITICAL: Common Ground. The battery powers the motors through L298N, and the ESP32 is powered separately (via USB or its own supply). Even though they're powered separately, all GND (ground) pins across the whole system must be wired together: battery GND → L298N GND, and L298N GND → ESP32 GND. Without this shared ground reference, the ESP32's control signals (IN1/IN2/IN3/IN4/ENA/ENB) will not make electrical sense to the L298N, and your motors will behave erratically or not at all.

## B.4 Full Wiring Table

ESP32 to L298N:

| ESP32 Pin | L298N Pin | Purpose |
| --- | --- | --- |
| GPIO 27 | IN1 | Left motor direction A |
| GPIO 26 | IN2 | Left motor direction B |
| GPIO 25 | IN3 | Right motor direction A |
| GPIO 33 | IN4 | Right motor direction B |
| GPIO 14 | ENA | Left motor speed (PWM) |
| GPIO 12 | ENB | Right motor speed (PWM) |
| GND | GND | Common ground — essential |

ESP32 to Servos (SG90):

| Servo | Signal wire → ESP32 | Power |
| --- | --- | --- |
| Servo 1 (Kicker Left / Flipper 1) | GPIO 13 | 5V + GND (see power note below) |
| Servo 2 (Kicker Right / Flipper 2) | GPIO 4 | 5V + GND (see power note below) |

Servo power note: SG90 servos can draw current spikes that a single ESP32 5V pin may struggle with, especially with two servos plus two DC motors active. For a bench-test/classroom build, powering servos from the ESP32 5V pin works fine for light loads (SG90 is a small/light servo). If you notice ESP32 resets or brownouts when servos move, power the servos from the same battery line as the motors (through a separate 5V regulator/UBEC) instead, keeping signal wires going to ESP32 GPIO and grounds all tied together.

ESP32 to HC-SR04 (Ultrasonic, front-mounted):

| HC-SR04 Pin | ESP32 Pin |
| --- | --- |
| VCC | 5V |
| GND | GND |
| Trig | GPIO 5 |
| Echo | GPIO 18 (through voltage divider — see B.5) |

Battery:

| Battery | Connects to |
| --- | --- |
| + (positive) | L298N 12V/VMS terminal |
| − (negative) | L298N GND terminal (and this GND rail also connects to ESP32 GND) |

ESP32 power: power the ESP32 either via its USB cable (fine for bench testing) or via its VIN pin from a regulated 5V source. Do not feed the raw 7.4–9V battery voltage directly into ESP32 5V/VIN without a regulator — it can damage the board. The simplest safe classroom approach: keep ESP32 powered via USB from a power bank taped to the chassis, while the battery pack powers only the L298N/motors. Just remember: grounds of the power bank/ESP32 and the motor battery must still be tied together for the control signals to work.

## B.5 Voltage Divider for HC-SR04 Echo Pin (Do This Properly Here)

The HC-SR04 Echo pin outputs 5V pulses; ESP32 GPIOs are rated for 3.3V max. Build a simple 2-resistor divider:

HC-SR04 Echo ---[1kΩ]---+---[2kΩ]--- GND

|

ESP32 GPIO18

- One end of the 1kΩ resistor connects to HC-SR04 Echo pin.

- The other end of the 1kΩ resistor connects to BOTH the 2kΩ resistor AND the ESP32 GPIO18 pin (this junction point).

- The other end of the 2kΩ resistor connects to GND.

This divides the 5V signal down to approximately 3.3V (5V × 2kΩ/(1kΩ+2kΩ) ≈ 3.3V), which is safe for the ESP32 GPIO. Trig pin doesn't need this — it's an ESP32 OUTPUT going to HC-SR04, already at safe 3.3V logic level which HC-SR04 reliably reads as HIGH.

## B.6 Mechanical Assembly

- Mount the two DC motors on the left and right sides of the chassis, wheels attached, facing the same direction.

- Mount the castor wheel at the front (or back — whichever end doesn't have the kicker) for balance; it should touch the ground and roll/swivel freely in all directions.

- Mount the L298N driver centrally, screws or double-sided tape, with easy access to its terminals.

- Mount the ESP32 in a location protected from wire clutter, ideally elevated slightly on standoffs.

- Mount the two SG90 servos at the front, arranged so their horns/arms can swing to strike a ball (this is your "kicker" — exact mechanical design of the kicker arm is flexible; a simple lever arm attached to each servo horn works well).

- Mount HC-SR04 facing forward, at roughly ball height, for detecting the ball/obstacles ahead.

- Route wires neatly, securing with zip-ties or tape so nothing dangles into the wheels.

- Mount the battery pack, keeping its weight centered for balance.

## B.7 The Full RoboSoccer Firmware (With Built-In Web Controller)

This single sketch does everything: creates the ESP32's own WiFi network, hosts a mobile-friendly control webpage, drives both motors (forward/back/left/right/stop), moves both kicker servos, and shows live ultrasonic distance readings that auto-update on the page.

Copy this entire code into a new Arduino IDE sketch.

/* ============================================================

ESP32 RoboSoccer Bot - Full Firmware with Web Controller

Prayas 4.0 - IIT Mandi

============================================================

Hardware: ESP32 DevKit, L298N, 2x DC motors, 2x SG90 servo,

1x HC-SR04 ultrasonic sensor, 1x castor wheel

============================================================ */

#include <WiFi.h>

#include <WebServer.h>

#include <ESP32Servo.h>

// ---------- 1. WIFI SETTINGS - CHANGE THESE IF YOU LIKE ----------

const char* ap_ssid     = "RoboSoccer_Bot";   // <-- CHANGE bot WiFi name here

const char* ap_password = "12345678";          // <-- CHANGE bot WiFi password here (min 8 chars)

// ---------- 2. PIN DEFINITIONS ----------

// Left motor (Motor A)

#define IN1 27

#define IN2 26

#define ENA 14

// Right motor (Motor B)

#define IN3 25

#define IN4 33

#define ENB 12

// Servos

#define SERVO1_PIN 13

#define SERVO2_PIN 4

// Ultrasonic

#define TRIG_PIN 5

#define ECHO_PIN 18

// ---------- 3. PWM (speed) SETTINGS ----------

const int pwmFreq = 1000;

const int pwmResolution = 8;      // 8-bit -> values 0-255

const int leftChannel = 0;

const int rightChannel = 1;

int motorSpeed = 200;             // default speed, 0-255 (adjustable from webpage)

WebServer server(80);

Servo servo1;

Servo servo2;

// ================= MOTOR CONTROL FUNCTIONS =================

void stopMotors() {

digitalWrite(IN1, LOW); digitalWrite(IN2, LOW);

digitalWrite(IN3, LOW); digitalWrite(IN4, LOW);

}

void moveForward() {

digitalWrite(IN1, HIGH); digitalWrite(IN2, LOW);

digitalWrite(IN3, HIGH); digitalWrite(IN4, LOW);

}

void moveBackward() {

digitalWrite(IN1, LOW); digitalWrite(IN2, HIGH);

digitalWrite(IN3, LOW); digitalWrite(IN4, HIGH);

}

void turnLeft() {

// left motor backward, right motor forward -> spin left

digitalWrite(IN1, LOW); digitalWrite(IN2, HIGH);

digitalWrite(IN3, HIGH); digitalWrite(IN4, LOW);

}

void turnRight() {

// left motor forward, right motor backward -> spin right

digitalWrite(IN1, HIGH); digitalWrite(IN2, LOW);

digitalWrite(IN3, LOW); digitalWrite(IN4, HIGH);

}

// ================= ULTRASONIC =================

long readDistanceCM() {

digitalWrite(TRIG_PIN, LOW);

delayMicroseconds(2);

digitalWrite(TRIG_PIN, HIGH);

delayMicroseconds(10);

digitalWrite(TRIG_PIN, LOW);

long duration = pulseIn(ECHO_PIN, HIGH, 30000);

if (duration == 0) return -1; // no echo received (out of range)

return duration * 0.034 / 2;

}

// ================= WEBPAGE (HTML+CSS+JS) =================

String htmlPage() {

String html = R"HTML(

<!DOCTYPE html>

<html>

<head>

<title>RoboSoccer Bot Controller</title>

<meta name="viewport" content="width=device-width, initial-scale=1, user-scalable=no">

<style>

body { font-family: Arial, sans-serif; background:#111; color:#fff; text-align:center; margin:0; padding:10px; }

h1 { color:#0f0; font-size:22px; }

.distance { font-size:20px; color:#0ff; margin:10px 0; }

.row { display:flex; justify-content:center; margin:8px 0; }

button {

width:90px; height:70px; margin:6px; font-size:16px; font-weight:bold;

border-radius:12px; border:none; background:#0a84ff; color:white; touch-action:manipulation;

}

button:active { background:#065bb5; }

.stop { background:#e63946; }

.kick { background:#ff9500; width:120px; }

.speedbox { margin:20px auto; width:80%; }

input[type=range] { width:100%; }

</style>

</head>

<body>

<h1>RoboSoccer Bot Controller</h1>

<div class="distance">Distance: <span id="dist">--</span> cm</div>

<div class="row">

<button ontouchstart="send('F')" ontouchend="send('S')" onmousedown="send('F')" onmouseup="send('S')">FORWARD</button>

</div>

<div class="row">

<button ontouchstart="send('L')" ontouchend="send('S')" onmousedown="send('L')" onmouseup="send('S')">LEFT</button>

<button class="stop" onclick="send('S')">STOP</button>

<button ontouchstart="send('R')" ontouchend="send('S')" onmousedown="send('R')" onmouseup="send('S')">RIGHT</button>

</div>

<div class="row">

<button ontouchstart="send('B')" ontouchend="send('S')" onmousedown="send('B')" onmouseup="send('S')">BACKWARD</button>

</div>

<div class="row">

<button class="kick" onclick="send('K1')">KICK 1</button>

<button class="kick" onclick="send('K2')">KICK 2</button>

</div>

<div class="speedbox">

<label>Speed: <span id="spdval">200</span></label>

<input type="range" min="100" max="255" value="200" id="spd" onchange="setSpeed(this.value)">

</div>

<script>

function send(cmd) {

fetch('/cmd?c=' + cmd);

}

function setSpeed(v) {

document.getElementById('spdval').innerText = v;

fetch('/speed?v=' + v);

}

function updateDistance() {

fetch('/distance').then(r => r.text()).then(d => {

document.getElementById('dist').innerText = d;

});

}

setInterval(updateDistance, 1000); // refresh distance every 1 second

</script>

</body>

</html>

)HTML";

return html;

}

// ================= WEB SERVER ROUTE HANDLERS =================

void handleRoot() {

server.send(200, "text/html", htmlPage());

}

void handleCmd() {

String c = server.arg("c");

if (c == "F") moveForward();

else if (c == "B") moveBackward();

else if (c == "L") turnLeft();

else if (c == "R") turnRight();

else if (c == "S") stopMotors();

else if (c == "K1") {

servo1.write(90);   // kick position - ADJUST ANGLES TO MATCH YOUR KICKER ARM

delay(300);

servo1.write(0);    // return position

}

else if (c == "K2") {

servo2.write(90);

delay(300);

servo2.write(0);

}

server.send(200, "text/plain", "OK");

}

void handleSpeed() {

String v = server.arg("v");

motorSpeed = v.toInt();

ledcWrite(leftChannel, motorSpeed);

ledcWrite(rightChannel, motorSpeed);

server.send(200, "text/plain", "OK");

}

void handleDistance() {

long d = readDistanceCM();

server.send(200, "text/plain", String(d));

}

// ================= SETUP =================

void setup() {

Serial.begin(115200);

// Motor pins

pinMode(IN1, OUTPUT); pinMode(IN2, OUTPUT);

pinMode(IN3, OUTPUT); pinMode(IN4, OUTPUT);

stopMotors();

// PWM setup for speed control (ESP32 LEDC peripheral)

ledcSetup(leftChannel, pwmFreq, pwmResolution);

ledcAttachPin(ENA, leftChannel);

ledcSetup(rightChannel, pwmFreq, pwmResolution);

ledcAttachPin(ENB, rightChannel);

ledcWrite(leftChannel, motorSpeed);

ledcWrite(rightChannel, motorSpeed);

// Servos

servo1.attach(SERVO1_PIN);

servo2.attach(SERVO2_PIN);

servo1.write(0);

servo2.write(0);

// Ultrasonic

pinMode(TRIG_PIN, OUTPUT);

pinMode(ECHO_PIN, INPUT);

// WiFi Access Point

WiFi.softAP(ap_ssid, ap_password);

Serial.println("=================================");

Serial.print("Connect your phone to WiFi: ");

Serial.println(ap_ssid);

Serial.print("Then open this in your browser: http://");

Serial.println(WiFi.softAPIP());

Serial.println("=================================");

// Web server routes

server.on("/", handleRoot);

server.on("/cmd", handleCmd);

server.on("/speed", handleSpeed);

server.on("/distance", handleDistance);

server.begin();

Serial.println("Web server started!");

}

// ================= LOOP =================

void loop() {

server.handleClient();

}

## B.8 Installing the ESP32Servo Library (Required Before Uploading)

The code above uses #include <ESP32Servo.h>, a library not installed by default. Install it like this:

- In Arduino IDE, go to Sketch → Include Library → Manage Libraries... (or click the library icon on the left sidebar).

- In the search box, type ESP32Servo.

- Find the library named "ESP32Servo" by Kevin Harrington / John K. Bennett.

- Click Install.

- Close the Library Manager and return to your sketch.

No other libraries need manual installation — WiFi.h and WebServer.h come bundled with the ESP32 board package you installed in Section A.4.

## B.9 Uploading the Firmware

- Connect ESP32 via USB.

- Confirm Tools → Board → ESP32 Dev Module and correct Tools → Port are selected (see Section A.4 if unsure).

- Before powering the motors/battery, upload the code first with just USB connected (safer — avoids driving motors accidentally during upload).

- Click Upload. If it hangs at "Connecting...", use the BOOT+EN button trick (Section A.2).

- Once uploaded, open Serial Monitor at baud 115200. You should see:

Connect your phone to WiFi: RoboSoccer_Bot

Then open this in your browser: http://192.168.4.1

Web server started!

- Now connect the battery to the L298N (motors will now have power).

- On your phone, connect to WiFi "RoboSoccer_Bot", password "12345678" (or whatever you changed them to).

- Open your phone's browser and go to 192.168.4.1.

- You should see the RoboSoccer Bot Controller page with Forward/Backward/Left/Right/Stop buttons, two Kick buttons, a speed slider, and a live distance readout.

## B.10 How the Code Works — Walkthrough

- WiFi Access Point: WiFi.softAP(ap_ssid, ap_password) makes the ESP32 create its own network — same concept from Section A.9.

- Motor direction: moveForward(), moveBackward(), turnLeft(), turnRight(), stopMotors() each set the 4 IN pins HIGH/LOW in the correct combination — same concept from Section B.3.

- Motor speed (PWM): ledcSetup() / ledcAttachPin() / ledcWrite() are ESP32's built-in PWM functions (called "LEDC" — LED Controller peripheral, reused here for motor speed). We configure two independent PWM channels, one for each motor's EN pin, and ledcWrite(channel, value) sets speed from 0 (stopped) to 255 (full speed).

- Servos (kicker): servo1.write(angle) moves a servo to a specific angle (0–180°). In handleCmd(), sending K1 swings Servo 1 to 90° (kick) then back to 0° (rest) after a short 300ms delay — adjust these angles/delay to match your physical kicker arm design.

- The webpage (htmlPage()): this is one big HTML string sent to the browser, styled with embedded CSS, containing buttons and a slider, plus a small block of JavaScript.

- How button presses reach the motors: Each button in the HTML calls the JavaScript function send('F') (or 'B'/'L'/'R'/'S'/'K1'/'K2'). That function does fetch('/cmd?c=F') — a background web request to the ESP32 asking for the URL path /cmd with a parameter c=F. On the ESP32 side, server.on("/cmd", handleCmd) catches this, handleCmd() reads the parameter with server.arg("c"), and runs the matching motor function. This round trip (button tap → phone sends tiny request → ESP32 receives it, acts, replies "OK") typically takes well under a tenth of a second on a direct AP connection, giving responsive real-time control.

- Press-and-hold movement: Forward/Back/Left/Right buttons use ontouchstart/ontouchend (for phone touchscreens) and onmousedown/onmouseup (for testing on a laptop trackpad) so the bot moves only while you're pressing the button and stops the instant you lift your finger — this is much safer and more intuitive for a soccer bot than toggle-style buttons.

- Live distance: the JavaScript setInterval(updateDistance, 1000) calls /distance once per second, and the ESP32's handleDistance() handler runs readDistanceCM() fresh each time and sends back just the number as plain text, which JavaScript drops into the page without reloading the whole page (smoother than the <meta refresh> trick used in Section A.12).

- Speed slider: dragging it calls setSpeed(value) → /speed?v=value → handleSpeed() updates motorSpeed and re-applies it to both PWM channels immediately.

## B.11 Changing WiFi Name/Password (Recap — Exactly Where to Edit)

Right near the top of the sketch:

const char* ap_ssid     = "RoboSoccer_Bot";   // <-- CHANGE bot WiFi name here

const char* ap_password = "12345678";          // <-- CHANGE bot WiFi password here (min 8 chars)

- Change only the text inside the quotation marks.

- ap_ssid is the WiFi network name your phone will see and connect to.

- ap_password must be at least 8 characters (ESP32 AP requirement) — fewer characters will fail to create the network.

- After changing, re-upload the code, then reconnect your phone to the new network name on your phone's WiFi settings (your phone won't automatically switch — you have to select it manually since it's a different network now).

## B.12 Testing Procedure (Step-by-Step, Safe Order)

- Bench test motors individually first, wheels off the ground (prop the chassis up on a box) — confirm Forward/Backward/Left/Right all spin the correct wheels the correct way. If a wheel spins the wrong direction, swap that motor's two wires on the L298N OUT terminals (this reverses it) rather than editing code.

- Test servos next, unloaded (no kicker arm attached yet) — confirm K1/K2 buttons move each servo and it returns to rest correctly.

- Test ultrasonic — confirm the distance readout on the page changes sensibly as you move your hand in front of the sensor.

- Attach kicker arms to servos, retest K1/K2 with the arm attached — adjust the 90/0 angle values in handleCmd() if the arm's rest/kick positions don't look right for your mechanical design.

- Place bot on the ground, retest Forward/Backward/Left/Right with wheels now touching the floor.

- Adjust speed slider to find a comfortable driving speed — very high speed on a small light chassis can make it hard to control precisely.

## B.13 Troubleshooting Guide

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| Can't find "RoboSoccer_Bot" WiFi on phone | Code didn't upload/run, or AP password <8 chars | Recheck Serial Monitor for "Web server started!"; ensure password length ≥8 |
| Webpage won't load in browser | Wrong IP typed, or phone not actually connected to bot's WiFi | Confirm phone WiFi settings show "Connected"; re-check IP via Serial Monitor (192.168.4.1 in almost all cases) |
| Buttons visible but robot doesn't move | Battery not connected to L298N, or common ground missing | Check battery connection; check GND wire from L298N to ESP32 GND is actually connected |
| One wheel doesn't spin | Loose motor wire, or that IN pin miswired | Recheck wiring against Section B.4 table |
| Wheel spins the wrong direction | Motor wires reversed relative to expected direction | Swap that motor's two wires at the L298N OUT terminal (physical swap, not code) |
| Motors very weak / barely turn | Battery low, or speed slider set low, or ENA/ENB not wired | Charge/replace battery; check ENA/ENB wiring; raise speed slider |
| ESP32 resets/reboots when motors or servos run | Power brownout — servos/motors pulling too much current from a shared/weak supply | Power servos from a separate 5V line off the battery (see B.4 servo power note); use a fresh/higher-capacity battery; add a large capacitor (e.g. 470–1000µF) across L298N power input |
| Ultrasonic always reads -1 or 0 | Wiring wrong, or Echo voltage divider missing/wrong values | Recheck Trig/Echo pins and voltage divider resistor values from Section B.5 |
| Distance readout frozen/not updating | Phone lost WiFi connection, or JavaScript blocked | Reconnect phone to bot WiFi; try a different browser (Chrome recommended) |
| Upload fails / "Connecting......." timeout | Board didn't enter flash mode automatically | Use BOOT+EN button trick from Section A.2 |
| Kicker servo moves too far / hits ground or frame | Kick angle too large for your arm design | Reduce the 90 value in handleCmd()'s K1/K2 blocks to a smaller angle |

## B.14 Customization Ideas (Once the Base Bot Works)

- Add a second ultrasonic sensor at the back (reuse the pattern from Section A.12/B.5 on new GPIO pins) for reverse obstacle detection, and add a second handleDistance2() route + second <span> on the webpage.

- Add a goal-detection color sensor or IR sensor as an additional input, following the same "read sensor → add web route → display on page" pattern used for the ultrasonic sensor.

- Adjust kick strength by changing servo kick angle and hold delay in handleCmd().

- Add a "Turbo" button on the webpage that temporarily sets motorSpeed to 255 for a burst of speed, then reverts.

- Switch to Station mode (Section A.8) instead of AP mode if you want the bot and your phone both on a shared hotspot/router network — useful if multiple people need to view the same bot's page, or if you want the bot to also reach the internet for logging.

## Summary

You've now gone from an unopened ESP32 board to a fully functional, phone-controlled RoboSoccer bot: you learned the hardware buttons, set up Arduino IDE with the ESP32 board package, uploaded and debugged your first sketches, used Serial communication, connected the ESP32 to WiFi both as a client and as its own access point, briefly saw Bluetooth as a contrast, learned to host a webpage with interactive controls and live sensor data, and finally combined all of that with an L298N motor driver, two DC motors, two servos, and an ultrasonic sensor into one complete, driveable, kickable RoboSoccer bot — all controlled from a single webpage in your phone's browser, no app required.